# flood_detection.py — Deep Dive Explanation

## Overview

This is a **flood detection and early warning system** that processes multi-temporal SAR (Synthetic Aperture Radar) satellite imagery to detect water bodies, track changes over time, and generate risk reports. It's a comprehensive remote sensing pipeline combining computer vision, deep learning (SAM2), and LLM-assisted analysis.

__________________________________________________

## Architecture & Data Flow

__________________________________________________

## Key Functional Sections

### 1. Configuration & Paths (lines 33-57)

Sets up file paths for SAM2 (Segment Anything Model 2)

Default output directory: `output/`

Pixel area for area calculation: 25 m²/pixel

LLM configuration (OpenRouter API with fallback model)

Window parameters: 512×512 with 128-pixel overlap for SAM2

### 2. Mask Caching (lines 97-136)

**Purpose:** Avoid re-running expensive SAM2 segmentation on the same images

Saves compressed numpy arrays (`.npz`) to disk

On subsequent runs, checks if cache exists and shape matches

**Significant performance optimization** for re-analysis

**Cache functions:**

`mask_cache_path()`: Generates cache file path

`save_mask_cache()`: Persists segmentation results

`load_mask_cache()`: Attempts to load cached results with validation

### 3. Image Preprocessing (lines 142-240)

**`load_all_images()`**:

Loads multiple TIF files using `rasterio`

Aligns all images to smallest common dimensions

Extracts metadata (transform, CRS, dimensions)

**`detect_nodata()`**:

Uses flood-fill algorithm (`cv2.floodFill`) to identify image edges with no data

Scans for zeros near borders and fills contiguous regions

Identifies radar "blind spots" in the imagery

**`enhance()`**:

Applies triple enhancement pipeline:

**Percentile stretching** (2%-98%): Normalizes dynamic range

**Log transformation** (`log(x+1)`): Enhances dark regions

**Gamma correction** (power=0.35): Improves contrast

**`guided_filter()`**:

Edge-preserving smoothing using box filters

Formula: `output = a * I + b` where `a` handles edges, `b` handles smooth areas

**`build_water_prob()`**:

Core water detection algorithm:

Applies guiding filter to enhanced image

Uses **Otsu thresholding** on filtered image

Applies stricter threshold (60% of Otsu value, max 45)

Detects **deep water**: values ≤8 with backscatter >0

Excludes **shadows** using local gradient analysis

- Calculates morphological gradient (dilation - erosion)

- Shadows have low gradient values

Generates probability map with three levels:

- `1.0` = definitive water (deep water or high-confidence)

- `0.2` = shadow/ambiguous water

- `0.0` = land or no data

Applies Gaussian blur for smooth probability distribution

### 4. SAM2 Segmentation (lines 272-359)

**Why SAM2?** Meta's Segment Anything Model 2 provides state-of-the-art promptable segmentation with minimal training data.

**How it works:**

**Sliding window approach**:

- Window size: 512×512 pixels

- Overlap: 128 pixels

- Ensures coverage of full image

**Cosine blending**:

- Each pixel weighted by cosine window

- Avoids tile artifacts at boundaries

- Formula: `weight = cos²(distance / size * π)`

**Point prompts**: Samples points from probability map:

- 6 foreground points (high prob > 0.5)

- 4 background points (low prob < 0.15)

- Minimum distance constraint: 20 pixels between samples

**SAM2 prediction**:

- `SAM2ImagePredictor.set_image()` on grayscale→RGB converted patch

- `predictor.predict()` with multimask_output=True

- Selects best mask by score

**Voting mechanism**:

- Each window's result weighted by prediction confidence

- Accumulates votes across all windows

- Final mask = votes / total_weights > 0.5

**Fallback**: If SAM unavailable or score < 0.3, uses probability threshold directly

**Key parameters:**

`WINDOW_SIZE = 512`

`OVERLAP = 128`

`N_FG = 6` (foreground points)

`N_BG = 4` (background points)

`SAM2_SCORE_MIN = 0.3`

**`init_sam2_predictor()`**:

Checks for SAM2 installation and checkpoint

Builds model with hybrid architecture (Hiera Small)

Detects CUDA availability for GPU acceleration

### 5. Post-Processing (lines 325-336)

**Purpose:** Removes noise while preserving true water features.

### 6. Feature Extraction (lines 370-407)

Extracts shape statistics for each disconnected water body:

Using OpenCV contour analysis:

**Contours and areas**: Detected via `cv2.findContours()`

**Perimeter**: Calculated with `cv2.arcLength()`

**Bounding boxes**: Minimum rectangle enclosing each contour

**Centroids**: Using image moments (`cv2.moments()`)

**Shape metrics:**

**Compactness** (circularity): `4πA/P²`

- Measures how close shape is to a circle

- Range: 0 (irregular) → 1 (perfect circle)

**Elongation**: `max(width, height) / min(width, height)`

- Aspect ratio of minimum-area rotating rectangle

- Higher = more elongated (e.g., rivers)

**Returns:**

### 7. Change Detection (lines 601-649)

Compares consecutive time steps to detect water movement:

**Algorithm:**

Masks aligned to valid intersection region (pixels valid in both)

For each consecutive pair:

- `persistent = mask_T1 & mask_T2`

- `receding = mask_T1 & ~mask_T2`

- `new = ~mask_T1 & mask_T2`

Calculates area in km² for each category

Generates RGB overlay visualization:

- Blue: persistent water (80% weight)

- Red: receding water (red tint)

- Green: new water (green tint)

- Gray: nodata regions

### 8. Quality Assessment (lines 413-459)

Computes segmentation accuracy metrics:

**Metric definitions:**

**IoU** (Intersection over Union): `TP / (TP + FP + FN)`

**Precision**: `TP / (TP + FP)` (proportion of detected water that is correct)

**Recall**: `TP / (TP + FN)` (proportion of true water detected)

**F1 Score**: `2 × (Precision × Recall) / (Precision + Recall)`

**Area Accuracy**: `1 - |pred_area - ref_area| / ref_area`

**Input handling:**

Ground truth masks loaded if provided

Otherwise uses high-threshold probability map as pseudo-reference

Parallel evaluation via `ThreadPoolExecutor` (up to 8 workers)

**Aggregation:**

Computes mean across all input images for each metric.

### 9. LLM Integration (lines 462-549)

**Two expert analysis modes:**

#### 1. Quality Review (`llm_quality_review`)

**Purpose:** Evaluates if segmentation meets quality standards

**Input:** Quality metrics (IoU, F1, Precision, Recall, area accuracy)

**LLM Prompt Structure:**

**Output:**

**Fallback:** Rule-based scoring when LLM unavailable:

Score = 0.35×IoU + 0.35×F1 + 0.15×Precision + 0.15×Recall

Verdict = "通过" if score ≥ 0.75 and area_accuracy ≥ 0.75

#### 2. Warning Expert (`llm_warning_expert`)

**Purpose:** Generates flood risk analysis and recommendations

**Input:** Full warning report stub (trends, risk assessment, etc.)

**LLM Prompt Structure:**

**Output:**

**API Configuration:**

Provider: OpenRouter

Model: Google Gemini 2.5 Pro (configurable)

Fallback: Rule-based proxy analysis

### 10. Warning Report Generation (lines 689-766)

Computes comprehensive risk assessment and generates structured report.

**Risk Score Formula:**

**Warning Levels:**

| Score | Level |
| --- | --- |
| ------- | ------- |
| ≥ 0.7 | High (高) |
| ≥ 0.4 | Medium (中) |
| < 0.4 | Low (低) |

**Impact Prediction:**

**JSON Structure (ver 1.0):**

### 11. Website Generation (lines 772-791)

Generates interactive HTML dashboard via `flood_web.write_dashboard()`.

**Features:**

Before/after slider comparison for time-series analysis

Area statistics visualization

Change detection overlays with color coding

Quality metrics dashboard

Responsive design for various screen sizes

**Data passed:**

### 12. Main Pipeline (`run_pipeline`, lines 797-869)

**Orchestrates the entire workflow:**

**Key behaviors:**

Graceful degradation when SAM2 unavailable

Automatic caching for speed

Parallel quality evaluation

Multi-language support (Chinese labels)

__________________________________________________

## Key Code Patterns & Design Decisions

| Pattern | Purpose | Example |
| --- | --- | --- |
| --------- | --------- | --------- |
| **Context manager** | Temporarily change directory for imports | `with switch_dir(SAM2_ROOT):` |
| **Global state** | Shared area reference per pipeline | `global PIXEL_AREA_M2` |
| **Fallback semantics** | Graceful degradation | SAM2 → probability map |
| **Caching strategy** | Per-label compressed storage | `{label}_mask_cache.npz` |
| **Cosine window** | Avoids tile artifacts | `win = outer(hanning(t), hanning(t))` |
| **Parallel processing** | Efficient metric computation | `ThreadPoolExecutor(max_workers=8)` |
| **LLM dual mode** | External + rule-based fallback | `external or llm_quality_review()` |

__________________________________________________

## Dependencies

__________________________________________________

## SAM2 Model Details

**Architecture:** Hierarchical Masked Autoencoder (Hiera Small)

-Checkpoint: `sam2.1_hiera_small.pt`

Config: `sam2.1_hiera_s.yaml`

**Why Hiera?**

Efficient hierarchical vision transformer

Good performance with smaller model size

Suitable for inference on CPU/GPU

**Device detection:**

__________________________________________________

## Usage Examples

### Command Line Interface

### Python API

__________________________________________________

## CLI Arguments Reference

| Argument | Type | Default | Description |
| --- | --- | --- | --- |
| ---------- | ------ | --------- | ------------- |
| `--input-files` | list | None | Specific TIF files to process |
| `--input-dir` | str | ROOT_DIR | Directory to scan for .tif/.tiff files |
| `--labels` | list | None | Custom labels for each image |
| `--ground-truth-files` | list | None | Reference masks for quality evaluation |
| `--out-dir` | str | "./output" | Output directory for results |
| `--pixel-area-m2` | float | 25.0 | Area represented by one pixel (m²) |
| `--disable-sam2` | flag | False | Skip SAM2, use probability segmentation only |
| `--force-rerun` | flag | False | Ignore cache, re-segment everything |

__________________________________________________

## Output Files

| File | Format | Description |
| --- | --- | --- |
| ------ | -------- | ------------- |
| `{label}_mask_cache.npz` | NumPy compressed | Cached water mask (reusable) |
| `{label}_prob_map.png` | Image | Water probability visualization |
| `{label}_seg_only.png` | Image | Water overlay only (for slider) |
| `{label}_water_result.png` | Image | Full visualization with legend |
| `change_{label1}_{label2}.png` | Image | Change detection overlay |
| `summary.png` | Image | 2×n grid of all results |
| `warning_report.json` | JSON | Structured analysis report |
| `index.html` | HTML | Interactive dashboard |

__________________________________________________

## Workflow Diagram

__________________________________________________

## Performance Characteristics

| Operation | Notes |
| --- | --- |
| ----------- | ------- |
| SAM2 inference | ~0.5-2 sec per 512×512 window (GPU) |
| Probability map | ~0.1-0.5 sec per image |
| Cache hit | Near-instant (np.load + shape check) |
| Full pipeline (3 images, SAM2) | ~2-10 minutes depending on size |
| Without cache | Each image re-segmented |

__________________________________________________

## Quality Control Points

**Cache validation**: Shape must match to prevent misuse

**Minimum area**: 300 pixels to filter noise

**SAM2 score threshold**: 0.3 minimum confidence

**Valid region intersection**: Ensures comparable analysis

**Ground truth fallback**: Pseudo-reference if GT unavailable

**LLM sanity checks**: Validated JSON output parsing

__________________________________________________

This is a production-grade remote sensing pipeline that balances:

**Accuracy**: SAM2 + probability fusion

**Efficiency**: Caching + parallel processing

**Explainability**: Visualizations + LLM analysis

**Robustness**: Fallbacks at every critical step